# Industrial NTP Time Server — Selection Sheet (Reference · Policy · Capacity)

| Item | Content |
| --- | --- |
| Audience | Agents / end customers / tender package |
| Document title | Selection Sheet (Reference · Policy · Capacity) |
| Doc No. | DOC-TS3000-SEL-EN-011 |
| Model | TempoSync-3000-BG-T-S |
| Document version | 1.1 (this respin: hardware 1PPS, PULSE PPM; no IRIG / no OCXO) |
| Firmware | V4.9.0 (see device Web / label) |
| Use | Selection / bid / acceptance (status — policy — capacity one-pager) |
| Companion | Shipping Product Specification Product Specification (Shipping) in this folder |

## 1. Reference source status

Time reference = GNSS 1PPS + disciplined local clock (not PTP; not dual-clock failover).

| Device state (Web / LCD) | Meaning | NTP under factory policy |
| --- | --- | --- |
| LINK up | Ethernet link OK | NTP / Web possible |
| GNSS fix / satellites | Module positioning valid | Discipline in progress |
| Warm-up (synced + PPS, not yet LOCK) | Cold-start converge | May reply Stratum 2 |
| LOCK | Reference ready | Stratum 1 |
| Brief PPS loss (after LOCK) | PPS blip window (~≤10 s) | May still reply |
| Unlock / holdover | GNSS or discipline lost | See §2 |
| No LINK | Cable / switch fault | No NTP |

Do not mix layers: internal/1PPS = ns parameters only; Ethernet NTP = lab clause (LOCK + low RTT → ms-class offset reachable; see report), no arbitrary-field guarantee; Modbus resolution 1 ms.

| Channel | This SKU | Accept |
| --- | --- | --- |
| RJ45 NTP | Stratum 1 after LOCK | LAN client; not module-ns |
| RS-485 Modbus | UTC incl. ms | FC03 regs 10–16 |
| 1PPS OUT | 硬件缓冲 GNSS-edge fan-out (3.3 V) | Scope 1 Hz (probe the panel 1PPS jack only) |
| PULSE (4-pin Pin4) | 200 ms at UTC minute after LOCK (5 V) | Scope; mute if unlocked |
| Not included | IRIG-B, OCXO, PTP, dual Ethernet | Pick another SKU for timecode / µs holdover |

## 2. NTP serve policy (Web)

| Web name | LCD Pol | Warm-up | After LOCK | After GNSS unlock |
| --- | --- | --- | --- | --- |
| Lock-based (factory default) | LCK | May S2 | S1 | Stop (except brief PPS blip) |
| Lock + timed hold | TIM | Same | S1 | Still S2 for ~≤5 min (preferred for Web export / soak) |
| Holdover serve | HLD | Sync → reply, usually S2 | Usually S2 | Continues (legacy “always need packets”) |

- Saved on Network page; persists across reboot.

- Prefer Lock + timed hold during large Web CSV/HTML exports.

- Optional NTP ACL is independent of the table above.

## 3. Capacity (parameters + formula)

| Boundary | Spec | Note |
| --- | --- | --- |
| NTP performance | Overload protect ≤ 2048 req/s; design reply capacity ≈ 1500 req/s (V-N10, 16 sources); (R) from same report | Parameters only; do not copy 10k-class appliance figures |
| Planning cap | 60 | Sales/SNMP; ≠ “must fill 60 at high poll” |
| Client count | (N(60,R/f_{})) | () 0.5–0.7; customer computes from poll rate |
| Per-source IP | ≤ 8 replies/s | Anti-abuse |
| During large Web export | Global cap further derated | Shared single port with NTP |
| Not included | IRIG-B, OCXO, PTP (no third-party PTP license pack), dual-port failover, IPv6, NTP broadcast/multicast, NTS, HTTPS, SNMPv3 | Entry industrial NTP (T = TCXO); SNMP is v2c read-only |

Acceptance: After LOCK, LAN w32tm / sntp OK; Web NTP Served increases; do not accept “module ns” as “client ns”; offset per lab/contract method only.

## 4. Quick answers

| Question | Answer |
| --- | --- |
| When is official Stratum 1? | LOCK under lock-class policy |
| Cold start with no time for clients? | Default allows warm-up S2; use timed hold for unlock behavior |
| How many clients? | Compute from formula (planning cap 60) — not a 10 k+ req/s appliance |
| Where does this sit? | Entry cabinet NTP — not PTP / not 10k-class room appliances |

Filename Selection Sheet. Interfaces: Industrial IF Protocol; operations: User Guide. Firmware as shown on device.